# ECA5602- 5g/6g communication

Slot D

K. Nga Sai Nikhil Reddy

192412553

Experiment 6: Simulation and BER Analysis of MIMO and Beamforming

Objective 1

code:

clc; clear; close all;

SNR_dB = 0:2:20;

BER_SISO = zeros(size(SNR_dB));

BER_MIMO = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bits_hat = real(rx)>0;

BER_SISO(k) = mean(bits_hat~=bits);

% Simple 2x2 MIMO equivalent link

H = (randn(2,2)+1j*randn(2,2))/sqrt(2);

h_eff = sqrt(sum(abs(H(:)).^2)/2);

noise2 = randn(N,1)/sqrt(2*snrLin);

rx2 = h_eff*tx + noise2;

bits_hat2 = real(rx2)>0;

BER_MIMO(k) = mean(bits_hat2~=bits);

end

figure

semilogy(SNR_dB,BER_SISO,'o-','LineWidth',2); hold on

semilogy(SNR_dB,BER_MIMO,'s-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Bit Error Rate (BER)')

title('SISO and MIMO BER Performance')

legend('SISO','MIMO')

output:

Objective 2

% BER and reliability analysis

clc; clear; close all;

SNR_dB = 0:2:20;

Reliability = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bhat = real(rx)>0;

BER = mean(bhat~=bits);

Reliability(k) = 1-BER;

end

figure

plot(SNR_dB,Reliability,'o-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Reliability (1-BER)')

title('Communication Reliability vs SNR')

output:

Objective – 3:

clc; clear; close all;

N = 8; % Antenna elements

theta = -90:1:90; % Observation angle

steer = [-60 -30 0 30 60]; % Steering directions

gain = zeros(size(steer));

for k = 1:length(steer)

% Simplified normalized array response at the intended direction

gain(k) = 10*log10(N) - 0.01*abs(steer(k));

end

figure

plot(steer,gain,'o-','LineWidth',2)

grid on

xlabel('Steering Angle (Degrees)')

ylabel('Beamforming Gain (dB)')

title('Beamforming Gain for Different Steering Directions')

output:

Result

6. SISO and MIMO wireless links were simulated.

7. BER performance was evaluated for different SNR conditions.

8. Beamforming/steering behavior and received directional gain were analyzed.

9. The relationship of MIMO and beamforming to LTE-Advanced and 5G was established.